Calculations

II.1. DENSITY

Speaking about density ......

All ASTM Tables require a density or equivalent (Relative Density or API) as argument to enter the tables.
In East European countries density at 20 oC is used. In West European countries density at 15 oC is often used.
In other countries an API or even a relative density at 60/60 oF is applied.

II.1.1. Density at t oC.

mass of a given volume of that product

The density of a product at t °C.  =  --------------------------------------------------------

Volume of that quantity at t°C.

Mass is given in Kg, volume given in litres or m3. So the density is expressed in kg/l or kg/m3,

taking in account that 1 kg/l = 1000 kg/m3.
Density is also referred to as: density at t oC.

Density appears in the following forms:
- density at 15°C.
- density at 20°C.

II.1.2. Relative density

Relative density occurs under the following names:

relative density
specific gravity 

The relative density t1/t2 °C. (or °F.) of a product is the ratio of the density of that product at t1 °, divided by the density of water   at t2 °. 

In formula:

density of product at t1°

relative density t1/t2°  =  -------------------------------------

density of water at t2°

Both above and below the bar of division the unit is kg/l, so the units will disappear in this formula. 

Conclusion: Relative density is a numeric value without unit.

II.1.2.1. Specific Gravity 60/60oF.

This is the old name for the relative density 60/60 °F. 

Meaning:

density of product at 60°

relative density 60 /60° = ------------------------------------

density of water at 60°

II.1.2.2.

The relative density 15/4 °C. =

density of product at 15°

relative density 15/4° = -------------------------------------

density of water at 4°

Since the density of water at 4 °C. is 1, as per definition, the value of the relative density 15/4 °C. is numeric equal to the density     at 15 °C.

II.1.3. Apparent density

This name is used for the density at t °C., corrected for the buoyancy of the displaced air. 
(So called 'density in AIR')

This apparent density is calculated can be found by means of W.C.F. table 56. This table is to be entered with the density at t °C. 

The result is the Weight Correction Factor Table 56 or Apparent density. (Note that for mineral products the difference

is 0.0011 kg/l.)

For chemical products with a density higher than 1.13 the difference gets less, see EPCA Blue book page 68.

II.1.4. API - Gravity

A.P.I. - gravity is a unit, which is used among others for calculations of Metric or Longtons from US Barrels 60 oF. 
API can be calculated from Relative Density 60/60 oF. by the following formula:

141.5

API = ---------------------------------------   - 131.5

Rel.Dens. 60/60 oF.

Of the above mentioned units ONLY the density at t °C. can be used for calculating mass ("weight"-in-VACUO) out of volume.

II.1.5. Weight Correction Factors

Relative density or API cannot be used for direct weight - calculation. Calculations should then be done using a weight correction factor (W.C.F.) All these calculations, using a W.C.F., result in a weight in AIR. 

These W.C.F.'s can be found in:

ASTM Table 11 (long tons per barrel 60oF.)
ASTM Table 13 (metric tons per barrel 60oF.)
ASTM Table 29 (longtons per barrel 60oF.)
ASTM Table 56 (kg/m3) (main table of 56)

II.2. MASS AND WEIGHT

ISO Nomenclature

-------------------------------------------------------------------------------------------------------------------

ISO NAME	SPEAKING

-------------------------------------------------------------------------------------------------------------------

Quantity in Kg		Mass 	Weight - in - vacuo

Quantity in Kg 		Weight 	Weight - in - air

Mass / Volume 		Density at °C. 	Spec.Grav.t/4°C. in vacuo

Weight / Volume 	W.C.F. 	Spec.Grav.t/4°C. in air

-------------------------------------------------------------------------------------------------------------------

Note: 		Longtons exist ONLY in Air!!!!!

II.2.1. Mass

The mass of a matter is the quantity of material that this matter exists of. 
This matter is independent from external conditions like attractive power of the earth (place on earth). 
In the oil business this is often mentioned as "VAC", or "Weight-in-vacuo".

Mass is calculated as follows:
Mass = Volume at t°C. x density at t°C.
(in kg or metric tonnes in VAC)

II.2.2. Weight

The weight of a product is the weight of that matter weighted in air, taking in account the buoyancy of the displaced air. Often related to as "Weight in Air". 

Weight calculation:

Metric system:
Weight = volume in m3 15oC. x w.c.f. (table 56)
(in kg or metric tonnes in AIR) 

Barrel system:
Weight = volume in US Bbls 60oF. x w.c.f. (table 11) 
(in longtons in AIR)

Weight = volume in US Bbls 60oF. x w.c.f. (table 13)
(in metric tons in AIR)

II.3. ASTM CALCULATIONS

The basis of all these calculations is that a volume measured at a certain temperature is corrected to a standard temperature by means of a volume correction factor. This corrected volume is called Standard Volume. 

From this standard volume SAYBOLT calculates Mass or Weight by means of a Weight Correction Factor of a Density at this standard temperature.

In the common used oil calculations two systems are used:

ACTUAL 	VOLUME 		STANDARD 		WCF 	WEIGHT

VOLUME	CORRECTION 		VOLUME 			(air)

API  ────┬─ TABLE 6A/B/D ───────── BARRELS 60°F ──┬──────	TABLE 11	LONG TONS

│			    	     │

BARRELS        │			      └──────	TABLE 13 	METRIC TONS

│

RD 60/60  ─┴─ TABLE 24A/B/D ──────── BARRELS 60°F ─────────	TABLE 29	LONG TONS

DENS.   ──┐

│

METRIC ──┴─ TABLE 54A/B/D ───────── LITRES 60°C ────┬──────	TABLE 56	METRIC TONS

│ ..................................................

│ 		(mass)

└──────	DENS. ─────  METRIC TONS

In general SAYBOLT calculates from volume into mass/weight as follows:

Volume and temperature of the product in a vessel or shore tank are measured. This measured volume (Total Observed Vo-ume) has to be corrected for trim, list, free water, floating roof etc. This corrected volume is called the Gross Observed Volume. This is the volume that will be corrected to a volume at a standard temperature; the Gross Standard Volume.

This standard volume can be expressed in:

Barrels at 60 oF.
Cubic meters at 15 oC.
(Cubic metres at 12 oC.)
(Cubic meters at 20 oC.)

Be aware that the correct volume unit is used together with the belonging calculating system.
for instance:
barrels together with API or RD 60/60
litres with Density at 15oC.
(If calculations are to be performed metric in vacuo, and the observed volume is given in barrels, this should be converted to liters by means of ASTM table 1.) 

This calculation from Observed volume to Standard volume is done by means of a Volume Correction Factor or V.C.F..
Next the obtained standard volume is multiplied by the density, resulting in mass, or multiplied by W.C.F., resulting in weight.

Volume correction factors are divided into three groups, depending on the kind of product:


ASTM Volume Correction Factors
------------------------------------------------
Tables 6-24-54 A : For crude oils 
Tables 6-24-54 B : For generalized products; fuel oil, gas oil, kerosene, gasoline, naphtha's
Tables 6-24-54 D : For lubricating oils


Tables 6 A-B-D give Volume Correction Factors for correcting Barrels at actual temperature to Barrels at 60 oF., depending on API at 60oF. and temperature in oF.

Tables 24 A-B-D give Volume Correction Factors for correcting Barrels at actual temperature to Barrels at 60 oF., depending on Relative Density 60/60oF. and temperature in oF.

Tables 54 A-B-D give Volume Correction Factors for correcting Litres at actual temperature to Litres at 15 oC., depending on Density at 15oC. and temperature in oC.

Examples:

Gas Oil (metric - vacuo calculation)

Density at 15 oC. (vacuo) 			0.8421 	Kg/l

Gross Observed Volume 			6,541,980 	litre

Temperature 				16.3 	oC.

V.C.F. to 15 oC. (54B) 			0.9989

Gross Standard Volume (6,541,980 x 0.9989) 		6,534,784 	ltr 15oC.

Mass (Weight-in-Vacuo) (6,534,784 x 0.8421) 		5,502,942 	kg

5,502.942 	Mtons

Crude Oil (barrel - calculation)

API at 60 oF. 				32

Gross Observed Volume 			543,234 	Barrels

Temperature 				68.4 	oF.

V.C.F. to 60 oF. 			0.9961

Gross Standard Volume (543,234 x .9961) 		541,115 	Bbls 60oF.

Table 11 				0.13512 	Ltons/Bbls 60oF.

Weight (in AIR)(541,115 x 0,13512) 		73,115.46 	Longtons

(For more examples see paragraph II.8)

II.4. CONVERSIONS

Saybolt makes a difference between conversions and calculations. 
A Calculation is the basic calculation from Observed Volume (eventually via Standard Volume) to Mass or Weight, depending on local regulations.

These basic calculations can be followed by Conversions, if necessary. Conversions must be regarded as translations from figures to other systems, necessary for comparison to other figures.
Conversion factors can be found in ASTM Tables, based on density, API or relative density or fixed factors (ASTM Table 1).

Volume conversions must be based on volume!

Mass or weight conversions must be based on mass or weight!

Calculation	     Conversions

┌────────────────┐

│ Observed Volume  │ ────► to actual volume conversions

└────────────────┘	       barrels/galllons/cubic feet

│   		       litres/cubic metres

│

▼

┌────────────────┐

│ Standard Volume   │ ────► to standard volume conversions

└────────────────┘	       barrels/galllons at 60°F

│   		       litres/cubic metres at 15°C

│

▼

┌────────────────┐

│    Mass or Weight   │ ────► to mass/weight corrections:

└────────────────┘	       vac / air / longtons

II.4.1. Air -> Vacuo

Whenever comparisons are made between different figures (air to vac or the other way around), these figures should both be converted to either air or vacuo. 
When the Bill of Lading is one of the figures to be compared, the conversions should be made towards the Bill of Lading figure. 
(The Bill of Lading figures should never be converted!)

These conversions should be done by means of the short tables of Table 56 (Saybolt Petroleum Tables Volume I, page 52).

(Table 56 consists out of a short table, to be used with conversions from vacuo to air or vice versa, and a main table, giving the W.C.F. in kg/m3 used to calculate weight in metric tons AIR out of volume [litres or m3]).

Example: 
A vessel arrives with a parcel of 30,000.260 mtons in VAC. Outturn density 0.8430 at 15 oC. The Outturn was 29,990.867 mtons in AIR. 
Since the outturn is established in AIR a conversion from air to vacuo is needed to make a good comparison between Outturn and Bill of Lading. In order to do so a conversion factor is needed to convert the Outturn from AIR to VAC. This conversion factor can be found in the short table of table 56, using the density at 15 oC as entry. In the table a correction factor of 1.00125 is found. The outturn in VAC can now be calculated by multiplying :

29,990.867 x 1.00125 = 30,028.356 Mtons in VAC.

II.4.2. Longtons <-> Metric Tons

These conversions are to be done by means of ASTM Table (Saybolt Petroleum Tables Volume I, page 21).

A Bill of Lading that has been achieved in Barrels and Longtons points to a Bill of Lading in AIR. When metric tons are converted out of the longtons (using ASTM Table 1) this will also be a figure in AIR. 

(ASTM Table 1; Longtons x 1.01605 = Metric Tons AIR).

Whenever figures are compared, it is necessary to make sure that the figures have the same unit. (E.g.: Mtons VAC to Mtons VAC, Longtons to Longtons, etc.)

Longtons can be calculated out of metric tons AIR, using the fixed factor 0.984206 from ASTM Table 1. Conversion from Metric tons in VAC to Longtons should be done as follows:

- convert the Metric tons in VAC to Metric tons in AIR, using the short table 56. 
- now convert the Metric tons in AIR to L-ong-tons using the factor 0.984206 from ASTM table 1.

II.4.3. API - Relative density - Density

With a lot of crude parcels API is used to calculate the Bill of Lading. For calculations in Europe a density is often required. 

This conversion can be done using ASTM Table 3. Enter this table with API at 60 oF., and then read the matching density at 15 oC. and if necessary the Relative density 60/60 oF. 

In the same way ASTM table 51 can be used to convert Density at 15 oC. to Relative density 60/60 oF. and API.

ASTM table 21 can be used to convert Relative density 60/60 oF. to API at 60 oF. or Density at 15 oF.

Example: 
Bill of Lading - 2,058,484 U.S. Barrels/60 oF. 
275,425 Longtons

Outturn - 280,150.300 Metric Tons (vac)
API/60 oF. : 33.60

To make a comparison between Bill of Lading and the Outturn the following should be done. 
- Convert API 60 oF to Density at 15 oC. 
- ASTM Table 3: API = 33.6 => Density = 0.8566
- Use density at 15 oC to find conversion factor from Vac to Air.
- ASTM Table 56: Density = 0.8566 => Conversion factor = 0.99875.

- Outturn in VAC to be multi-plied by factor table 56:

280,150.300 x 0.99875 = 279,800.112 Metric tons AIR

- Use ASTM Table 1 for conversion from Mtons to Longtons: 0.984206

- 279,800.112 x 0.984206 = 275,380.95 Ltons 

Never compare air to vac or vice versa!

II.4.4. Volume Conversions

Conversions from one volume unit to another (e.g. barrels 60°F. to litres 15°C.) should be done on volume basis: 

Barrels 60°F x corr. factor = litres 15 °C.

II.4.4.1. Cubic metres 15°C  to Barrels 60oF

This conversion is done by one of the ASTM tables:
Table 4, 21, 22, or 52. Which of these tables must be used is dependent on whether calculations are based on density or API.

II.4.4.2. Barrels 60°F to Gallons 60°F

When US Barrels are being converted to US Gallons a fixed factor of 42 is used. (ASTM Table 1: 1 barrel = 42 gallons as per definition).

II.4.4.3. Cubic metres 15°C to Gallons 60°F.

When converting Cubic meters at 15 °C. into Barrels at 60 °F. and Gallons at 60 °F. this should be done as follows:

- multiply cubic meters at 15 °C. by the correction factor from table 52 (Barrels 60°F. per cubic meter 15°C.).

- then BEFORE rounding the barrel figure, multiply this by 42 in order to get the Gallons at 60°F.
- now both the Gallon and the Barrel figure can be rounded.

Example:
A vessel loaded the following Bill of Lading:
16,798,715 Litres at 15 oC.
14,772,790 Kg (Mass)
density: 0.8794

Following conversions are asked:
Litres at 15 oC.
US Barrels at 60oF.
US Gallons to 60oF.
Metric tons in vac and in air
Longtons

Calculation:
US Barrels 60oF: factor table 52 = 6.293
(16,798,715 : 1000) x 6.293 = 105,714.3-14
US Gallons 60oF: 42 x 105,714.3-14 = 4,440,001

US Barrels 60oF (rounded): 105,714
Metric Tons (AIR): Factor table 56 = 0.99875

(14,772,790 x 1000 x 0.99875 = 14,754.324)
Longtons: 14,754.324 x 0.984206 = 14,251.29

Factors and tables

from 		to  		ASTM table 	Factor

Barrels 60°F. 	Gallons 60°F. 	1 	42

Metric Tons Air 	Longtons 		1 	0.9842

Longtons (air) 	Metric Tons Air 	1 	1.0161

Entry Item

API 		RD 60/60 & Dens.15 	3 	API

Barrels 60°F. 	Litres 15°C. 		4 	API

RD 60/60°F. 		Density 15°C. 	21 	RD 60/60

Barrels 60°F. 	Litres 15°C. 		22 	RD 60/60

Density 15°C 	API & RD 60/60 	51 	Density

Litres 15°C. 		Barrels 60°F. 	52 	Density

Metric Tons Vac 	Metric Tons Air 	56 	Density

Metric Tons Air 	Metric Tons Vac 	56 	Density

II.5. GROSS & NET CALCULATIONS

GROSS STANDARD VOLUME:

The entire volume of all petroleum liquids, including Sediment and Water but excluding Free water, corrected by the appropriate volume correction factor for the measured temperature and density to a standard reference tem-perature (and in case of metering also corrected for the appropriate pressure correction factor and meter factor).

NET STANDARD VOLUME:

The entire volume of all petroleum liquids excluding suspended Sediments and Water and Free water cor-rected by the appropriate volume correction factor for the measured temperature and density to a standard reference temperature (and in case of metering also corrected for the appropriate pressure correction factor and meter factor).

II.7. PRODUCTS, LPG AND PETROCHEMICALS

The main difference between foregoing calculations and the next groups is that the in most cases only the crude calculations are divided into Gross and Net- Calculations. For products working with Net-calculations is not a common procedure.

II.7.1. Products

Calculation of refined products such as naphtha, gasoline, kerosene, jet fuel, gas oil and fuel oil does not differ much from the gross crude cal-culations, except that for all these products table 6B, 24B or 54B are used instead of the A-series.

For lubricating oils another set of tables has been developed: the D-series. (6D, 24D, 54D).

Another difference is that in product calculations only gross cal- culations are used. Net calculations are usually not performed while working with products.

Example 1:
Calculate the Mass of a parcel of gas oil with a density of 0.8421 kg/l. The Gross Observed Volume is 23,763,980 litres at a temperature of 17.5 oC.

G.O.V. = 23,763,980 litres
V.C.F. = 0.9979 x (Table 54B, density .8421 and temp. 17.5 oC)

G.S.V. = 23,714,076 litres at 15 oC
Density= 0.8421 x
Mass = 19,969.623 Mtons (Weight-in-Vac)

Example 2:
Calculate the Weight in Longtons of a parcel of fuel oil with an API of 18.5. The Gross Observed Volume is 138,344 US barrels at a temperature of 136.5 oF.

G.O.V. = 138,344 US Barrels
V.C.F.  = 0.9689 x (Table 6B, API 18.5 and temp. 136.5 oF)
G.S.V.  = 134,042 US Barrels at 60 oF
W.C.F. = 0.14729 x (Table 11, API 18.5)

Weight = 19,743.05 Longtons (Weight-in-Air)

II.8. CALCULATION COMPARISON

In this paragraph we show you various ways of calculations as used by SAYBOLT, and we show you the relation and differences between the different systems.

In these comparisons we use 25,000,000 liters of an oil with an actual density of 0.8380 kg/l at 25 oC. 
The same volume expressed in actual barrels : 25,000,000 x 0.00628981 = 157,245 barrels.

First we convert the actual density and temperature to: 
density at 15oC, 
relative density 60/60oF (by means of ASTM Table 51),
API gravity.

At the same time we use this example oil as a Crude Oil, Product, Lube Oil, and we also calculate this oil by means of the old table 53/54.

ASTM table 	Density 15/4 oC.  	Rel.dens 60/60oF.  	API gravity

A.S.T.M table 51 	A.S.T.M table 51

53 A 	0.8451 		0.8455 		35.85

53 B 	0.8449 		0.8453 		35.89

53 D 	0.8441 		0.8445 		36.09

53 	0.8447 		0.8451 		35.93

Gost Den.20/4 = 0.8416 corr./oC = 0.000712 = 0.8452 at 15/4 oC.

The volume correction factors will be looked for in the appropiate table. As equivalent for 25 oC. we use 77oF.

54A 	54B 	54D 	54 	6A 	6B 	24A 	24B

25oC. 	0.9914 	0.9916 	0.9925 	0.992

77oF.         					0.9918 	0.9921 	0.9919 	0.9921

Сalculations : Metric, Mass
Conversions : Air, Longtons, Barrels 60 oF., Gallons 60 oF.

Table 54 A 	Table 54 B

Calculation:

Litres at 25°C. 			25,000,000 	25,000,000

V.C.F. 			0.9914 	0.9916

Litres at 15°C. 			24,785,000 	24,790,000

Density at 15°C. 		0.8451 	0.8449

Kilos (vac) 			20,945,804 	20,945,071

Weight Conversions:

Kilos (vac)			20,945,804 	20,945,071

ASTM Table 56 (short table) 		0.99875 	0.99875

Kilos (air) 			20,919,622 	20,918,890

ASTM Table 1 			0.984206 	0.98421

Longtons 			20,589.22 	20,588.50

Volume Conversions:

Litres at 15°C. 			24,785,000 	24,790,000

ASTM Table 52 		6.293 	6.293

US Barrels at 60°F. 		155,972 	156,003(.47)

ASTM Table 1 			42 	42

US Gallons at 60°F.  		6,550,824 	6,552,146

Calculations : Metric, Air
Conversions : Vac, Longtons, Barrels 60 oF., Gallons 60 oF.

Table 54 A 	Table 54 B

Calculation

Litres at 25oC. 			25,000,000 	25,000,000

V.C.F. 			0.9914 	0.9916

Litres at 15oC. 			24,785,000 	24,790,000

W.C.F. ASTM Table 56 (Main table) 	0.844 	0.8438

Kilos (air) 			23,918,540 	20,917,802

Weight Conversions:

Kilos (air)     			23,918,540 	20,917,802

ASTM Table 1 			0.984206 	0.984206

Longtons 			20,588.15 	20,587.43

Kilos (air)     			20,918,540 	20,917,802

ASTM Table 56 (short table)  		1.00125 	1.00125

Kilos (vac)  			20,944,688 	20,943,949

Volume Conversions:

Litres at 15oC.     		24,785,000 	24,790,000

ASTM Table 52 		6.293 	6.293

US Barrels at 60oF. 		155,792(.00) 	156,003(.47)

ASTM Table 1 			42 	42

US Gallons at 60oF. 		6,550,824 	6,552,146

Calculations : Barrels, API gravity, via Table 11
Conversions : Metric tons Air, Vac, Litres15 oC., Gallons 60 oF

Table 6 A  	Table 6 B

Calculation

US Barrels 			157,245 	157,245

V.C.F. 			0.9918 	0.9921

US Barrels at 60oF. 		155,956 	156,003

W.C.F. ASTM Table 11 		0.13201 	0.132

Longtons 			20,587.75 	20,592.40

Weight Conversions:

Longtons 			20,587.75 	20,592.40

ASTM Table 1 			1.01605 	1.01605

Metric tons (air) 		20,918.183 	20,922.908

ASTM Table 56 (short table) 		1.00125 	1.00125

Metric tons (vac) 		20,944.331 	20949.062

Volume Conversions:

US Barrels at 60oF. 		155,955.59 	156,002.76

ASTM Table 4 			0.15891 	0.15891

Litres at 15oC. 			24,782,903 	24,790,399

US Barrels at 60oF. 		155,955.59 	156,002.76

ASTM Table 1 			42 	42

US Gallons at 60oF. 		6,550,135 	6,552,116

Calculations : Barrels, API gravity, via Table 13 
Conversions : Longtons, Vac, Litres 15 oC., Gallons 60 oF.

Table 6 A  	Table 6 B

Calculation

US Barrels 			157,245 	157,245

V.C.F. 			0.9918 	0.9921

US Barrels at 60oF. 		155,956 	156,003

W.C.F. ASTM Table 13 		0.13412 	0.13409

Metric tons (air) 		20,916.819 	20,918.442

Weight Conversions:

Metric tons (air) 		20,916.819 	20,918.442

ASTM Table 1 			0.984206 	0.984206

Longtons 			20,586.46 	20,588.06

Metric tons (air) 		20,916.819 	20,918.442

ASTM Table 56 (short table) 		1.00125 	1.00125

Metric tons (vac) 		20,942.965 	20944.59

Volume Conversions:

US Barrels at 60oF.		155,955.59 	156,002.76

ASTM Table 4 			0.15891	 0.15891

Litres at 15oC. 			24,782,903 	24,790,399

US Barrels at 60oF. 		155,955.59 	156,002.76

ASTM Table 1 			42 	42

US Gallons at 60oF. 		6,550,135 	6,552,116

Calculations : Barrels, Rel.density 60/60oF., via Table 29
Conversions: Metric tons Air Air,Vac, Litres15 oC.,Gallons 60 oF.

Table 24 A 	Table 24 B

Calculation

US Barrels 			157,245 	157,245

V.C.F. 			0.9919 	0.9921

US Barrels at 60oF. 		155,971 	156,003

W.C.F. ASTM Table 29 		0.132 	0.13197

Longtons 			20,588.17 	20,587.72

Weight Conversions:

Longtons 			20,588.17 	20,587.72

ASTM Table 1 			1.01605 	1.01605

Metric tons (air) 		20,918.610 	20,918.153

ASTM Table 56 (short table) 		1.00125 	1.00125

Metric tons (vac) 		20,944.758 	20,944.301

Volume Conversions:

US Barrels at 60oF. 		155,971.32 	156,002.76

ASTM Table 4 			0.15891 	0.15891

Litres at 15oC. 			24,785,402 	24,790,399

US Barrels at 60oF. 		155,971.32 	156,002.76

ASTM Table 1 			42 	42

US Gallons at 60oF.  		6,550,79 5 	6,552,116

Calculations : Metric, Vacuo Table 54D & Old table 54
Conversions: Metric tons Air, Longtons, Barrels 60 oF., Gallons 60 oF.

Table 54 D 	Old Table 54

Calculation

Litres at 25°C. 			25,000,000 	25,000,000

V.C.F. 			0.9925 	0.9920

Litres at 15°C. 			24,812,500 	24,800,000

W.C.F. ASTM Table 56 (main table) 	0.8441 	0.8447

Kilos (vac)     			20,944,231 	20,948,560

Weight Conversions:

Kilos (vac)     			20,944,231 	20,948,560

ASTM Table 56 		0.99875 	0.99875

Kilos (air)			20,918,051 	20,922,374

ASTM Table 1 			0.984206 	0.984206

Longtons  			20,587.67 	20,591.93

Volume Conversions:

Litres at 15°C.     		28,812,500 	28,800,000

ASTM Table 52 		6.293 	6.293

US Barrels at 60°F. 		156,145(.06) 	156,066(.40)

ASTM Table 1 			42 	42

US Gallons at 60°F.  		6,558,093 	6,554,789

Calculations : Metric, Gost
Conversions: Metric tons air, Longtons, Barrels 60 oF.,Gallons 60oF.

Gost

Calculation
Litres at 25oC 			25,000,000
Density at 25oC 		0.8380
Kilos (vac) 			20,950,000

Weight Conversions:
Kilos (vac)			20,950,000
ASTM Table 56 		0.99875
Kilos (air)			20,923,813	
ASTM Table 1			0.984206
Longtons			20,593.34
Volume Conversions:
Litres at 15oC			24,784,100
ASTM Table 52 		6.293
US Barrels at 60oF 		155,966(.34)
ASTM Table 1 			42
US Gallons at 60oF  		6,550,586

II.9. TERMINOLOGY

ASTM 			American Society for Testing Materials.

A.P.I. 			American Petroleum Institute.

I.P. 			Institute of Petroleum.

I.S.O. 			International Standardisation Organization.

Bill of Lading 			A document issued by the cargo supplier which states the quantity delivered

to the vessel.

Calibration table 		A table showing the volumes of liquid which a given tank holds at various liquid

levels measured from a reference point.

Dip (innage) 			The depth of a liquid measured from the surface of the fixed datumplate or to

the tank bottom.

Floating roof correction 		The correction which allows for the oil displaced by the floating roof.

(in the critical zone, accurate correction is not possible)

Free flowing liquid 		The liquid which is left in cargo tanks after unloading which is capable of free flow.

(ISO draft)

Free Water (FW) 		The measured volume of water present in a container which is not in suspension in

the contained liquid at observed temperature.

Gross standard density of oil 		The mass per unit Gross Standard Volume of oil.

Gross Standard Volume (GSV) 	The entire volume of all petroleum liquids and sediment and water, excluding free

water, corrected by the appropriate volume correction factor for the measured

temperature and density to a standard reference temperature.

Mass 			The mass of a substance is the quantity of matter it contains and as such is 			independent of buoyancy, changes in acceleration due to gravity and other external 			conditions.

Gross weight in air of oil 		The mass which a gross standard volume of oil has when weighed in air.

Indicated volume 		The change in meter reading that occurs during a receipt or delivery.

Indicated volume with factor 		The indicated volume multiplied by the meter factor for the particular liquid and

operating conditions under which the meter was proved.

In transit difference 		The difference between the Total Calculated Volume immediately after a loading and

the Total Calculated Volume immediately before the next unloading.

Line circulation 		Petroleum or other liquid delivered through a pipeline system into a receiving vessel or

tank to ensure that the section of pipeline from the source tank to the receiving tank

is full.

Line displacement 		Done when necessary to replace the material in a pipeline with a different material.

Line press (pack) 		The recorded difference in tanks gauges when a line is pressurized by pumping from

the tank into the closed pipeline system, and taking the second tank gauge while a

specific line pressure is maintained.

Line drop 			Opening (venting to atmosphere) a vessel's piping system so as to allow, to the extent

possible, their drainage into ship's tank(s) where the material may be gauged and

accounted for.

Load On Top (LOT) procedure 	This is the shipboard procedure of collecting and settling water and oil mixtures,

resulting from ballasting and tank opera-tions (usually in a special sloptank or tanks)

and the subsequent loading of cargo onto such mixtures.

Load on top (practice) 		The act of comingling on board quantity with cargo being loaded.

Meter factor 			The ratio of the actual volume of liquid passing through a meter to the volume

indicated by the meter.

Net Observed Volume (NOV) 		The entire volume of all petroleum liquids, excluding Sediment and Water and

Free Water at measured temperature and pressure.

Net standard density of oil 		The mass per unit Net Standard Volume of Oil.

Net Standard Volume (NSV) 		The entire volume of all petroleum liquids, excluding suspended Sediments and

Water and Free Water, corrected by the appropriate correction factor for the 			measured temperature and density to a standard reference temperature (and also 			corrected by the applicable pressure correction factor and meter factor.

Net Wt-in-Air of oil 		The mass which a Net Standard Volume of Oil has when weighed in air.

Notice of apparent discrepancy 	A letter issued by any participant in a custody transfer citing any condition with which

issue is taken. This serves as a written record that the particular action of finding was

questioned at the time of occurrence.

On Board Quantity (OBQ) 		All the oil, water, sludge and sediment in the cargo tanks and associated lines and 			pumps on a ship before loading a cargo commences.

Outturn quantity  		The quantity of oil expressed in Net Standard Volume received from a ship, on-shore

or off-shore installation during a given time or in a particular batch.

Outturn certificate  		A statement issued by a receiving party, certifying the Outturn Quantity.

Outturn loss/gain  		The difference, in Net Standard Volume, between the quantity shown on the Bill of 			Lading and the quantity shown on the Outturn

Certificate. 			It may be expressed as a volume or a percentage of the Bill of Lading quantity.

Part cargo 			A quantity of oil occupying less then 80 % of the available cargo capacity.

Quantity Remaining On Board (ROB) 	All the oil, water, sludge and sediment in the cargo tanks and associated lines and

pumps on a ship after discharging of a cargo has been completed, excluding vapours.

Reference level 		A fixed location on or in a tank from which measurements are made.

Dip measurements are usually referred to a firmly fixed level plate located on or close

to the tank bottom but may be referred to the tank bottom.

Slops 			Oil, oil/water emulsions, sediment and water contained in slop tanks or designed

cargo tanks.

Suspended water 		The water within the oil, commonly present as inhomogenously dispersed water

droplets of varying dimensions.

Tank gauging 			The measurement by dip or ullage.

Total Calculated Volume 		The entire volume of all petroleum liquids and suspended sediment and water,

(TCV = GSV + FW)		excluding free water, corrected by the appropriate volume correction factor for the

observed temperature and density to a standard reference temperature and also

corrected by the applicable pressure factor and meter factor with all free water 			measured at observed temperature and pressure added back in

(i.e. Gross Standard Volume + Free Water).

Total Observed Volume (TOV) 	The entire volume of material contained in a tank at the tank temperature.

Ullage 			The distance from an upper reference level to the air/liquid interface in a tank.

Vessel Experience Factor (VEF) 	The mean value of the Vessel Load Ratio's or Vessel Discharge Ratio's (VDRs) after 			several qualified voyages.

Vessel Load Ratio (VLR) 		The ratio of the Total Calculated Volume (TCV) measured on board a vessel

immediately after loading less the On Board Quantity (OBQ) to the TCV measured

by the loading terminal.

Vessel Discharge Ratio (VDR) 		The ratio of the Total Calculated Volume (TCV) measured on board a vessel

immediately before discharge less the quantity Remaining On Board (ROB) to

the TCV measured by the receiving terminal.

Vessel -shore difference 		The difference between the Total Calculated Volume recorded by the vessel

corrected for OBQ or ROB as appropriate and the Total Calculated Volume recorded

by the shore.

Volume Correction Factor (VCF)	A factor for correcting oil volumes to a standard reference temperature.

Water cut			The procedure of locating the oil/water interface for the purpose of determining the

volume of free water in a shore tank or vessel compartment.

Wedge formula 		A mathematical means to estimate small quantities of cargo and free water on board

prior to loading and after discharge based on cargo compartment dimensions and 			vessel trim. The wedge formula is to be used only when the liquid does not cover

the entire bottom of vessels tank. The volume of the wedge is expressed as a fraction

of the volume for the uncorrected dip at the end of the tank.

Weight Conversion Factor (W.C.F.) 	A factor for converting volumes to weight-in-air.